No. 94-006

q	Optional	TIMELINE VISTA, INC.

1755 LA COSTA MEADOWS DRIVE, SUITE B

q	Recommended	SAN MARCOS, CA   92069

Tel. 760-761-4440

q	MANDATORY	(FAX 760-761-4449)

SUPPORT@DIGAUDIO.COM

## NEVE/CCU Installation

### DATE:	07/25/94

### MODEL:	Console Control Unit

### REVISION:	All

### SERIAL NO:	All

### SOFTWARE:	CCU 230 and later
	SSU 140 and later

### DESCRIPTION:

An update is required in the Neve Flying Faders configuration file.  This update improves CCU performance in Neve/SSU installations.

### PROCEDURE:

Part 1:  Flying Faders Computer

Note:  We recommend that these changes be made by someone who is familiar with DOS computer systems.

1.	Exit from the Flying Faders program on the Flying Faders Computer, to the DOS prompt.

2.	With a text editor, add the following lines to the end of the Neve Configuration File:  C:\mfsys\mfad.ini.

## TimeLine CCU/SSU System Configuration

## Note:  This section must be removed if the SSU is disconnected.

## Enable Transport control

set tapeismast true

## Disable External Transport Control Buttons

tape rewind off

tape forward off

tape play off

tape stop off

Part 2:  CCU

1.	Turn off the CCU and remove the jumper between pins 1 and 2 of CCU Header J5.

2.	Power up the CCU while holding the CLR Key to reset the CCU.

3.	Under [SETUP] + [SYS] on the CCU, check that control Via is set to DIRECT.

Figure 1.  Jumper Location