# PROM Installation Guide

## Lynx Film Module

q	Optional	TIMELINE VISTA, INC.

1755 LA COSTA MEADOWS DRIVE, SUITE B

q	Recommended	SAN MARCOS, CA   92069

TEL. 760-761-4440

q	Mandatory 	(FAX 760-761-4449)

SUPPORT@DIGAUDIO.COM

### DATE:	06/04/92

### MODEL:	Lynx Film Module

### REVISION:	All

### SERIAL NO:	All

### SOFTWARE:	PROM FI__ , 	U15 (128K)

### REQUIRED TOOLS:

Static safe workstation	IC Extractor/Insertor or 
	Grounding wrist strap	small slotted screwdriver	
	Phillips screwdriver

### REQUIRED PARTS:

### Software Information Sheet
	Replacement PROM from TimeLine

### PROCEDURE:

1.	Turn off the power to the Lynx Film Module. Disconnect all cables.

2.	Place the Lynx on a static safe workstation. Ground yourself and the workstation 
anti-static mat.

3.	Remove the 6 phillips screws holding the top cover to the chassis. Remove 
the top cover.

Figure 1. Remove the Top Cover

## PROM Installation Guide

### PROCEDURE continued:

4.	Locate U15 on the Film Processor board.  Remove and replace the PROM. Be careful to correctly align Pin 1 and properly seat the PROM.

Warning

Use an IC inserter to remove and replace the PROM. If one is not available, use a small 
slotted screwdriver. Insert the screwdriver under the PROM, twist slightly and gently lift up. 
Inserting the screwdriver incorrectly could damage surrounding components.

Figure 2.  PROM Location

7.	Replace the cover on the Lynx Film Module. Insert and tighten the 6 phillips screws. Reconnect the cables and turn on the power. The Lynx module will automatically detect the new software version.